THE CARGO DESK   Capacity Reference

# Capacity Guide

Dimensions, weights and loadability across road, sea, air and breakbulk. Last updated 23 Apr 2026.

## Road  (7 units)

Australian road freight. Payloads assume compliant GVM and tare; practical payload is often reduced by driver, fuel, and pallet weight.

| 1-tonne Courier Van | Length (int.): 3.00 m Width: 1.50 m Height: 1.60 m Volume: 7.2 m³ Max payload: 1,000 kg Pallets: 2 AU / 2 Euro Sprinter/HiAce class. Tail-lift rarely fitted; hand-unload only. |
| --- | --- |
| 3-tonne Tail-Lift | Length (int.): 4.50 m Width: 2.10 m Height: 2.20 m Volume: 20.8 m³ Max payload: 3,000 kg Pallets: 6 AU / 8 Euro Light rigid truck. Ideal for urban last-mile; tail-lift standard. |
| 5-tonne Rigid | Length (int.): 6.00 m Width: 2.30 m Height: 2.40 m Volume: 33.1 m³ Max payload: 5,000 kg Pallets: 10 AU / 12 Euro Medium rigid truck. Most common metro distribution vehicle. |
| 8-tonne Rigid | Length (int.): 7.00 m Width: 2.40 m Height: 2.45 m Volume: 41.2 m³ Max payload: 8,000 kg Pallets: 12 AU / 14 Euro Heavy rigid. Common for full-load industrial and palletised LCL. |
| 12-tonne Rigid | Length (int.): 8.50 m Width: 2.40 m Height: 2.50 m Volume: 51 m³ Max payload: 12,000 kg Pallets: 14 AU / 16 Euro Large rigid, usually tautliner or pantech. Upper limit for most metro depots. |
| Semi-Trailer (Tautliner) | Length (int.): 13.60 m Width: 2.48 m Height: 2.70 m Volume: 91.1 m³ Max payload: 25,000 kg Pallets: 22 AU / 33 Euro Standard prime mover + 13.6 m trailer. 42.5 t GCM. Workhorse of AU interstate freight. |
| B-Double | Length (int.): 25.00 m Width: 2.48 m Height: 2.70 m Volume: 167.4 m³ Max payload: 42,000 kg Pallets: 44 AU / 66 Euro Two trailers linked by a fifth wheel. 62.5 t GCM. Permitted on approved B-double routes. |

## Sea  (10 units)

ISO shipping containers. Internal dimensions and payloads are typical; tare and max gross weight vary by build and carrier. AU road limits often cap actual payload below container max.

| 20' General Purpose | Length (int.): 5.90 m Width: 2.35 m Height: 2.39 m Volume: 33.2 m³ Max payload: 28,230 kg Max gross: 30,480 kg Tare: 2,200 kg Workhorse dry box. Practical AU road payload is ~21–22 t with tare/trailer. |
| --- | --- |
| 40' General Purpose | Length (int.): 12.03 m Width: 2.35 m Height: 2.39 m Volume: 67.7 m³ Max payload: 26,700 kg Max gross: 30,480 kg Tare: 3,750 kg Volume-friendly dry box. Road payload limit almost always reached before weight limit. |
| 40' High Cube | Length (int.): 12.03 m Width: 2.35 m Height: 2.69 m Volume: 76.4 m³ Max payload: 26,580 kg Max gross: 30,480 kg Tare: 3,900 kg Extra 30 cm of internal height. Default for volumetric cargo. Height-restricted at some AU sites. |
| 45' High Cube | Length (int.): 13.56 m Width: 2.35 m Height: 2.69 m Volume: 86 m³ Max payload: 27,600 kg Max gross: 32,500 kg Tare: 4,800 kg Pallet-wide variants common in Europe; less common in AU. Road permits often required. |
| 20' Reefer | Length (int.): 5.44 m Width: 2.29 m Height: 2.27 m Volume: 28.4 m³ Max payload: 27,700 kg Max gross: 30,480 kg Tare: 3,080 kg Integrated refrigeration unit reduces internal length and height. |
| 40' Reefer High Cube | Length (int.): 11.58 m Width: 2.29 m Height: 2.56 m Volume: 67.8 m³ Max payload: 29,200 kg Max gross: 34,000 kg Tare: 4,800 kg Standard for AU meat, dairy, and produce exports. |
| 20' Open Top | Length (int.): 5.90 m Width: 2.35 m Height: 2.37 m Volume: 32.8 m³ Max payload: 28,130 kg Max gross: 30,480 kg Tare: 2,300 kg Removable tarpaulin roof. For cargo loaded top-down by crane; over-height requires out-of-gauge. |
| 40' Open Top | Length (int.): 12.03 m Width: 2.35 m Height: 2.37 m Volume: 66.9 m³ Max payload: 26,500 kg Max gross: 30,480 kg Tare: 3,980 kg Longer members; same tarp principle as 20'. Common for machinery. |
| 20' Flatrack (Collapsible) | Length (int.): 5.94 m Width: 2.20 m Height: 2.23 m Max payload: 30,140 kg Max gross: 34,000 kg Tare: 2,360 kg No sides or roof. Collapsible ends for return stacking. For over-width, oversize, and heavy project cargo. |
| 40' Flatrack | Length (int.): 12.08 m Width: 2.40 m Height: 2.14 m Max payload: 40,000 kg Max gross: 45,000 kg Tare: 5,000 kg Long machinery, boats, transformers, prefab. Heaviest payload of any ISO unit. |

## Air ULDs  (7 units)

Unit Load Devices. Contours (slanted profiles) match the aircraft fuselage — lower-deck LD units have a chamfer, main-deck units are full rectangular prisms. Payloads are IATA-standard maxes; airline restrictions often apply.

| AKE / LD3 IATA AKE | Length (int.): 1.56 m Width: 1.53 m Height: 1.63 m Base: 156 × 153 cm Volume: 4.3 m³ Max payload: 1,587 kg (3,500 lb) Used on: Lower deck — B777, B787, A330, A350, B747 The standard lower-deck container. One side is chamfered to match the fuselage curve. |
| --- | --- |
| AKN / LD3-45 IATA AKN | Length (int.): 1.56 m Width: 1.53 m Height: 1.14 m Base: 156 × 153 cm Volume: 3 m³ Max payload: 1,134 kg (2,500 lb) Used on: Lower deck — B737, A320 series Shorter LD3 for narrow-body belly holds. |
| ALF / LD6 IATA ALF | Length (int.): 3.17 m Width: 1.53 m Height: 1.63 m Base: 318 × 153 cm Volume: 8.8 m³ Max payload: 3,175 kg (7,000 lb) Used on: Lower deck — B777, B747, A330, A350 Two LD3 footprints. Chamfered both long sides. |
| AAP / LD9 IATA AAP | Length (int.): 3.17 m Width: 2.23 m Height: 1.63 m Base: 318 × 224 cm Volume: 10.8 m³ Max payload: 4,626 kg (10,200 lb) Used on: Lower deck — B747, B777, A330 Full-width lower-deck container. Rectangular (no contour). |
| PMC / P6P Pallet IATA PMC | Length (int.): 3.17 m Width: 2.44 m Height: 1.63 m Base: 318 × 244 (96" × 125") cm Volume: 12.6 m³ Max payload: 6,804 kg (15,000 lb) Used on: Main & lower deck — most wide-bodies and freighters The most common air cargo pallet. Netted and strapped; contours shaped to aircraft on loading. |
| PAG / P1P Pallet IATA PAG | Length (int.): 3.17 m Width: 2.23 m Height: 1.63 m Base: 318 × 224 (88" × 125") cm Volume: 11.5 m³ Max payload: 4,626 kg (10,200 lb) Used on: Main & lower deck — wide-bodies Narrower pallet, commonly used for belly hold loading. |
| AMA / M-1 Main Deck IATA AMA | Length (int.): 3.17 m Width: 2.44 m Height: 2.44 m Base: 318 × 244 cm Volume: 17.6 m³ Max payload: 6,804 kg (15,000 lb) Used on: Main deck — B747F, B777F Full 8' × 10' × 8' main-deck container. Tall — requires freighter or combi aircraft. |

## Freighters  (6 units)

Most common commercial freighter aircraft in international service. Payload and volume are typical maxes; actual lift depends on range, fuel, and runway.

| Boeing 747-8F | Volume: 858 m³ Max payload: 137,700 kg Main deck: 30 × PMC + nose-load capability Lower deck: 14 × PMC/PAG or 2× AKE per position Cargo door: 3.05 m H × 3.40 m W (main-side + nose) Max single piece: 3.00 m H × 3.35 m W × 12.20 m L Typical range: 8,130 km Nose-loading door lets 12.2 m single pieces slide in straight. Flagship queen of air cargo. |
| --- | --- |
| Boeing 777F | Volume: 653 m³ Max payload: 102,800 kg Main deck: 27 × PMC Lower deck: 10 × PMC + 1 × LD3 Cargo door: 3.05 m H × 3.70 m W (main-side) Max single piece: 3.00 m H × 3.17 m W Typical range: 9,200 km Best payload/range of twin-engine freighters. Industry backbone; dominant on long-haul trunk lanes. |
| Boeing 767-300F | Volume: 438 m³ Max payload: 52,700 kg Main deck: 24 × LD2 / 19 × PMC positions Lower deck: 30 × LD2 equivalent Cargo door: 2.59 m H × 3.40 m W (main-side) Max single piece: 2.49 m H × 3.18 m W Typical range: 6,025 km Regional and express workhorse (FedEx, UPS, DHL). Narrower cross-section than 777. |
| Boeing 737-800BCF | Volume: 141 m³ Max payload: 23,900 kg Main deck: 11 × 88 × 125 pallet positions Lower deck: Bulk hold only Cargo door: 2.16 m H × 3.56 m W (main-side) Max single piece: 2.03 m H × 2.08 m W Typical range: 3,750 km Converted narrow-body. Huge in e-commerce and feeder networks (Amazon Air, ASL, China Post). |
| Airbus A330-200F | Volume: 475 m³ Max payload: 70,000 kg Main deck: 23 × PMC Lower deck: 8 × LD3 + 2 × LD6 or equivalent Cargo door: 3.00 m H × 3.58 m W (main-side) Max single piece: 2.97 m H × 3.56 m W Typical range: 7,400 km Medium-range widebody freighter. Common with Qantas Freight, Qatar, Turkish. |
| Airbus A321P2F | Volume: 158 m³ Max payload: 28,000 kg Main deck: 14 × 88 × 125 pallet positions Lower deck: 10 × LD3-45 Cargo door: 2.24 m H × 3.40 m W (main-side) Max single piece: 2.11 m H × 2.09 m W Typical range: 3,700 km Largest narrow-body freighter currently in service. Growing fleet in AU domestic networks. |

## Breakbulk  (6 units)

Multi-purpose, heavy-lift, and semi-submersible vessels for project cargo, breakbulk, and out-of-gauge units. Fleet examples drawn from BBC Chartering, Spliethoff / BigLift, Jumbo, AAL, and Boskalis. Crane figures are combined tandem lifts where vessels carry two cranes.

| MPP EuroCarrier (AAL / BBC F-type) | DWT: 9,300 t Cargo hold: 13,650 m³ Hold shape: Box-shaped, ~90 m × 16 m, 3 hatches Tween deck: Movable pontoons — 2 decks configurable Container slots: 558 TEU Tank-top load: 15 t/m² Cranes (2): 150 t single / 300 t tandem Max piece weight: 300 t Workhorse multi-purpose vessel. Boxed holds take 12 m, 40' and 45' containers; pontoons lift for tall pieces. BBC F-500/EuroCarrier class. |
| --- | --- |
| Heavy-Lift F-700 (BBC Chartering) | DWT: 12,700 t Cargo hold: 17,000 m³ Hold shape: Boxed, 2 hatches, ~104 m total hold length Tween deck: Pontoon tween deck Container slots: 681 TEU Tank-top load: 20 t/m² Cranes (2): 250 t single / 500 t tandem Max piece weight: 500 t Mid-range heavy lift. Tandem 500 t handles transformers, wind nacelles, pressure vessels. BBC F-700 class. |
| Project Heavy-Lift P-Type (BBC Pioneer) | DWT: 17,300 t Cargo hold: 23,600 m³ Hold shape: Boxed, 2 hatches, ~120 m × 20 m Tween deck: 3 pontoon positions Container slots: 880 TEU Tank-top load: 25 t/m² Cranes (2): 400 t single / 800 t tandem Max piece weight: 800 t Upper end of the merchant HL fleet. Tandem 800 t lifts cover most wind, oil & gas, and rail project work. |
| Spliethoff S-Type Multipurpose | DWT: 21,500 t Cargo hold: 26,900 m³ Hold shape: Full box holds, 3 hatches, ice class 1A Tween deck: Movable tween decks (6 positions) Container slots: 1,054 TEU Tank-top load: 20 t/m² Cranes (2): 120 t single / 240 t tandem Max piece weight: 240 t Spliethoff's S-class. High ice class and boxed holds make it a backbone of project and breakbulk trades on northern routes. |
| Super Heavy-Lift (Jumbo / BigLift) | DWT: 14,000 t Cargo hold: 17,000 m³ Hold shape: Open-top boxed hold, 1–2 hatches Tween deck: Lift-out pontoons Tank-top load: 30 t/m² Cranes (2): 900 t single / 1800 t tandem Max piece weight: 1,800 t Jumbo J-Class / BigLift Happy series. Mast cranes reach tandem 1,800 t lifts for refinery modules, offshore topsides, full jacket sections. |
| Semi-Submersible Heavy Transport (Boskalis / COSCO) | DWT: 76,000 t Hold shape: Open deck 178 m × 42 m (no hold — cargo sits on deck) Tween deck: n/a — open deck Tank-top load: 45 t/m² Max piece weight: 50,000 t Deck submerges so cargo floats or is skidded on. Used for drill rigs, FPSOs, naval vessels, decommissioning. Examples: Boka Vanguard (117 k DWT, 275 m × 70 m), COSCO Xin Guang Hua. |

Figures are typical industry values. Internal dimensions and payloads vary slightly by manufacturer, carrier tare, and national road limits. Always confirm with the carrier before booking.